NYISO

Initial Planning Process

Draft 8/21/03

LIPA Comments 8/27/03

NYISO Initial Planning Process

# Table of Contents

- Introduction

- Stakeholder Process

- Planning Criteria and Objectives

  - Reliability Criteria

    - NERC

    - NPCC

    - NYSRC

  - Objectives

    - Reliability Needs Assessment

    - Historic Congestion

- Process

  - Overview

  - Input Stage

    - Input Requirements

      - Load and Capacity Data Report

      - Existing Reliability Assessments

      - Short Circuit Data

    - Input from Neighboring Areas

      - Forecasted Load, Facilities and System Conditions

    - Input from Stakeholders

    - Baseline

    - Scenario Development

  - Analysis Stage

    - Historic Congestion

      - Summary Reports

      - Selective Cause Analysis

    - Baseline Reliability Needs Assessment

    - Evaluation of Alternate Scenarios

    - Report Preparation

  - Review Process

    - Stakeholder Review

    - Board Action

- Issuance of Final Report

  - Follow Up Discussions

- References

7.   Figures

Attachment A – Process Flow Chart

Attachment B – Stakeholder Participation

Attachment C -  Timeline

Add definitions section.  Below are suggested words for definition:

Neighboring Control Areas

Congestion

Short Circuit Assessment

Other sections to be added should include:

Reference to methodology to be used in the conduct of the study.

Will the NYISO Short Circuit Procedure be used for the Short Circuit Analyses using the NPCC database?

How will reliability needs study assessment be performed?

Consider including appropriate variant of the graphic from June 13 meeting (system_planning.pdf) – shown below – to demonstrate the input information and the steps that will be conducted to arrive at the System Plan.

How frequently will the Plan be developed?

- Introduction

The NYISO Initial Planning Process is the first phase in the development of a comprehensive planning process for the NYISO.  This process is being developed by NYISO Staff with assistance of the Electric System Planning Working Group (“ESPWG”), an ad- hoc committee comprised of BIC and MC representatives.  The Initial Planning Process will focus on the consolidation of the existing NYISO reliability-based analyses, an extension of those reliability analyses from 5 to 10 years, and the addition of scenario analyses to the base case conditions.  In addition, the Initial Planning Process will include the reporting of historical congestion costs and selective analysis of the causes of historic congestion in order to provide information to the marketplace to assist in future decision making.  (See Attachment A for the Process Flow Diagram depicting the major elements of this Initial Planning Process.)

It is anticipated that development efforts on the Comprehensive Planning Process (Phase II) will begin immediately upon the establishment of the Phase I process.  The Phase II discussions will address the various FERC planning requirements of Order 2000, the SMD NOPR, and the Wholesale Market Design Whitepaper, including both reliability and economic issues.  It is anticipated that a FERC filing will be required for the Comprehensive Planning Process.

- Stakeholder Process

In light of the fact that the Initial Planning Process contains both reliability and economic features, it has been agreed that both the Transmission Planning Advisory Subcommittee (“TPAS”) and the ESPWG will participate in the implementation process.  This participation will consist of parallel input and review stages as shown in Attachment B.

TPAS will have primary responsibility for the reliability-based analyses, while the ESPWG will have primary responsibility for providing commercial input and assumptions utilized in the development of future scenarios as well as provide input into the reporting and analysis of historic congestion costs.  Coordination will be established between these two groups and with NYISO Staff during each stage of the planning process.

The intention is to achieve consensus at both TPAS and the ESPWG.  While no formal voting process has been established at this level, an opportunity for reporting majority and minority views will be provided in the absence of a consensus.

Following TPAS and ESPWG final review, the NYISO Staff’s Final Draft Report will be forwarded to the Operating Committee (“OC”) .  See Section 4.4.1 for a further description of the Stakeholder Review Process.

- Planning Criteria and Objectives

The New York Control Area (“NYCA”) power system is planned and operated to the planning and operating policies, standards, criteria, guidelines, procedures and rules promulgated by the North American Electric Reliability Council (“NERC”), Northeast Power Coordinating Council (“NPCC”), and the New York State Reliability Council (“NYSRC”). NERC establishes operating policies and planning standards for North America which includes the United States of America and the Provinces of Canada. NPCC criteria, guideline and procedures which apply to the five areas comprising NPCC (New York State, the New England States, and the Canadian Provinces of Quebec, Ontario and the Maritimes) may be more specific or more stringent than NERC standards and policies by recognizing regional characteristics or reliability needs – e.g., “the one day in ten years” loss of load expectation criteria. The NYSRC rules that apply to NYCA may be more specific or stringent than NERC and NPCC by recognizing NYCA characteristics and reliability needs – e.g., locational capacity requirements. The NYISO is the primary interface between market participants and the reliability councils. The chart below presents an overview of those interfaces.

The objective of the initial planning process is four fold: 1) to demonstrate that the NYCA power system expansion plans are consistent with the reliability rules and will ensure the continued reliability (i.e., adequacy and security) of the power system consistent with good utility practice; 2) to identify, through scenario development, factors and issues that might adversely impact the reliability of the power system; 3) to provide, through the analysis of historical congestion  costs, information to market participants where the addition of system upgrades could potentially improve the economic efficiency of the power system; and 4) coordinate the reliability assessment with Neighboring Control Areas.

  - Reliability Criteria

    - NERC:  Establishes standards and policies for North America which includes the United States of America and the Provinces of Canada.  The NERC Operating Policies and Planning Standards can be found on the NERC web site at http://www.nerc.com/standards/.

    - NPCC: Establishes criteria, guidelines and procedures which apply to the five areas comprising NPCC (New York State, the New England States, and the Canadian Provinces of Quebec, Ontario and the Maritimes) may be more specific or more stringent than NERC standards and policies by recognizing regional characteristics or reliability needs. The NPCC criteria, guides, and procedures can be found on the NPCC web site at http://www.npcc.org/CriteriaGuidesProcedures.htm. (1)

    - NYSRC: Establishes rules that apply to NYCA that may be more specific or stringent than NERC and NPCC by recognizing NYCA characteristics and reliability needs. NYSRC rules can be found on the NYSRC web site at http://www.nysrc.org/documents.html. See Reliability Rules Revision No. 3 (adopted May 9, 2003). (2)

  - Objectives

    - Reliability Needs Assessment

The baseline system as defined for the Annual Transmission Reliability Assessment (ATRA) (3) will be assessed to determine if it meets all the reliability criteria for both resource and transmission adequacy. The ATRA focuses on the first five years of the planning horizon. Reliability needs will be defined in terms of quantities and not necessarily in terms of specific facilities. For instance, the MW quantity of additional transfer capability or MW quantity of additional resources would be specified.

In addition, the baseline system will be evaluated for robustness against factors and issues identified through scenario analysis that might adversely impact the reliability of the power system for years one through ten. This evaluation of the baseline system for robustness will only identify conditions under which the reliability of the power system might potentially be at risk [will the risk be quantified?  Or will the assessment just evaluate whether the criteria has been met?]. It will not identify or propose additional needs.

    - Historic Congestion

The NYISO will prepare summaries of historic congestion across the New York system.  In addition, the NYISO will conduct analyses to identify the significant causes of the historic congestion.

- Process

  - Overview

It has been stated that the planning process is as important as the plan itself, if not more important. This is certainly true for the initial planning process. The purpose of the initial planning process is to facilitate the exchange of information between the NYISO, Market Participants and interested stakeholders regarding the future reliability of the NYCA power system and the historical congestion performance of the transmission system. The expansion plans used in these reliability assessments will be those proposed by market participants. The key elements of the initial planning process are described further below.   See Attachment C for the proposed timeline for completion of the first study to be performed as part of the NYISO Initial Planning Process.  [should there be recognition here that the process may be refined during the execution of the first study to refine procedures, etc.?  If so, how will those modifications be approved by stakeholders?]

  - Input Stage

    - Input Requirements

The input to be used in the initial planning process includes: published data sources, existing standard reliability assessments, data for additional reliability analysis, input from Neighboring Control Areas, input from stakeholders, and input from the ESP Working Group regarding alternate scenarios.

      - Load and Capacity Data Report

The 2003 NYISO Load and Capacity Data Report [provide reference/link] will be the primary reference resource for generation, load, and transmission facilities for the initial planning study.

4.2.1.2    Existing Reliability Assessments

The existing standard reliability assessments that will be used in the initial planning study include:

- The 2003 NPCC New York Area Transmission Review (ATR) (4)

- The 2003 NPCC New York Resource Adequacy Review (RAR) (5)

- The 2003 NYSRC Installed Reserve Margin (IRM) Study (6)

4.2.1.3    Short Circuit Data

Ordinarily the initial planning study would use the information from the NYISO Annual Transmission Reliability Assessment (ATRA).  However, because the 2003 ATRA has been delayed for an indefinite period, it will be necessary for the initial planning study to include a separate Short Circuit assessment.  Therefore, for the initial planning study the short circuit database will be consistent with that of the NPCC New York Area Transmission Review [provide reference/citation].  [Does this representation have the Neighboring Control Areas?  Is this database in a sufficient form to perform the short circuit analysis in accordance with the NYISO SC procedure?]

    - Input from Neighboring Areas

The initial planning study will use the most recent power flow data collected through the annual regional (NPCC) and inter-regional (NERC) base case development process as the primary reference resource for the Neighboring Control Areas.

      - Forecasted Load, Facilities and System Conditions

The NYISO also will coordinate directly with the Neighboring Control Areas to exchange additional supplemental information for the study including: forecasted load, significant new or modified generation and transmission facilities, and anticipated system conditions.

    - Input from Stakeholders

In addition to information published in the NYISO Load and Capacity Data Report, the NYISO will solicit additional information for the initial planning study directly from stakeholders.  This input will include:

- Transmission system – existing and planned additions – Transmission Owners

- Merchant transmission proposals – Merchant developers

- Generation additions/retirements – Generation Owners & Developers

- Demand response programs – Demand Response Providers

- Long-term firm transmission service requests – NYISO and Transmission Owners

[what will be the criteria for inclusion of new resources?]

[Consider classifying projects as:  Under Construction, Planned, Proposed, Concept – could use definitions similar to those previously used by ISO-NE or PJM]

    - Baseline

The baseline will be developed based on the NYISO Load and Capacity Data Report, the existing reliability assessments, and the Neighboring Control Areas and stakeholders input.

    - Scenario Development

The ESP Working Group will provide input regarding alternate scenarios for additional reliability analyses.  Scenarios will be developed in two time frames: the first five years (years 1 through 5), and the next five years (years 6 through 10).  Variables for consideration in the development of these scenarios include:

- Load Forecast Uncertainty

- Fuel (prices and availability of supply)

- New Resources

- Retirements

- Transmission network topology (e.g., change in procedure of lines that are normally open; change in contingencies based on breakers being operated normally open or closed; etc.)

- Limitations imposed by environmental legislation

  - Analysis Stage

    - Historic Congestion

The NYISO will prepare summaries of historic congestion across the New York system.  In addition, the NYISO will conduct analysis to identify the significant causes of the historic congestion.  The process for measuring congestion is under development and will be defined in an attachment to be added to this document at a later date.

[Question:

Will historic congestion be analyzed as day ahead, real time, or combination?]

      - Summary Reports

The NYISO will prepare various reports of historic congestion costs.  These reports will be based upon the actual data from the NYISO day-ahead market, and will include summaries, aggregated by month [should also be able to provide summaries by hour of day, day of week, etc.], such as:

- By NYCA

- By zone

- By contingency in rank order

- By constraint in rank order

- Total congestion cost

- Congestion cost by facility

- $/Mwh [?]

- Hours of congestion by facility

- Congestion by major cause category

  - Construction

  - Forced outage

  - Maintenance

  - No outages

      - Selective Cause Analysis [what’s the criteria to “selectively analyze the causes”?

The NYISO will perform an analysis to identify the cause of significant congestion levels.  Such analysis will include the following elements:

- Identification of the cause of major transmission [and generation] outages.  The major categories would include construction, maintenance, and forced outages.

- Quantification of the impact on congestion costs for relieving significant system constraints [could you please explain/expand on this item?]

    - Baseline Reliability Needs Assessment

The NYISO will evaluate the reliability needs of the New York system for the first five-year and second five-year baseline.  The evaluation will address resource and transmission adequacy over both periods.  It also will address short circuit fault duty for the first five-year period.  The evaluation will be based on information from the existing standard reliability assessments (see §4.2.1) and additional reliability analysis will be performed to include the effects of input from Stakeholders and the Neighboring Control Areas for the second five year period.

The analyses for the baseline reliability needs assessment will first determine whether or not the baseline resources and transmission system would meet all applicable reliability criteria (per §3.1).  Then, if any reliability criteria would not be met, additional analyses will be conducted to determine the additional resources and/or transmission that would be needed to meet criteria, and to determine the first year of need for those additional resources and/or transmission.  For the initial planning process, these needs, if any will be expressed in terms of incremental capability needed [this implies that generation and transmission will be treated equal in potential for providing solution].  The study will not seek to identify specific additional facilities.

    - Evaluation of Alternate Scenarios (Robustness of Baseline)

After completion of the baseline reliability needs assessment, the NYISO will conduct additional reliability analyses for the alternate scenarios specified by the ESP Working Group.  These evaluations will test the robustness of the baseline needs assessment.  The reliability needs may increase in some scenarios and may decrease, or even be eliminated, in others.

    - Report Preparation

Once all the analyses have been completed, the NYISO will prepare a draft report of the results.  [can proposed table of contents for this report be provided?]

  - Review Process (See Attachment B)

    - Stakeholder Review:  At least two stakeholder review stages are anticipated in the Initial Planning Process.  Following review of the Staff’s Final Draft Report by TPAS and the ESPWG, it will be forwarded to the OC for a vote.  Interested representatives from the Business Issues Committee (“BIC”) are invited to attend the OC meeting at which the Final Draft Report is under consideration.  Following the OC vote, the Final Draft Report will be transmitted to the Management Committee (“MC”) for a vote.

    - Board Action

Following the MC vote, the Final Draft Report resulting from the NYISO Initial Planning Process will be forwarded to the NYISO Board for action.  Upon acceptance by the Board, the report will be finalized by NYISO Staff.

- Issuance of Final Report

Following Board acceptance, the NYISO Staff will issue the Final Report resulting from implementation of the NYISO Initial Planning Process to the marketplace.  This report will identify potential reliability needs determined under various future scenario assumptions and will provide reports and analyses of historic congestion costs.

This Report is intended to provide useful information to market participants as they consider alternative strategies for the future.

  - Follow-Up Discussions

In order to provide the maximum benefits to the marketplace for the information provided by the Initial Planning Process, the NYISO will provide various opportunities for market participants and other stakeholders to discuss the Final Report.  Such opportunities may include presentations at various NYISO stakeholder committees, focused discussions with various sectors, and/or presentations in other public venues.

- Disclaimer

[We need a disclaimer section.  Throughout the discussion of the ESPWG the ISO staff has said that this initial process does not determine the final process and that market participants and the NYISO Board are not giving up any rights or setting a legal precedent by adopting this initial process.  We should state this in a disclaimer as part of this document.]

- References

- NPCC Basic Criteria for Design and Operation of Interconnected Power Systems (A-2)

- NYSRC Reliability Rules For Planning and Operating the New York State Power System

- NYISO Open Access Transmission Tariff – Attachment S

- NPCC Guidelines for NPCC Area Transmission Reviews (B-4)

- NPCC Guidelines for Area Review of Resource Adequacy (B-8)

- NYSRC Policy 5

- Figures

Attachment A – Process Flow Chart

Attachment B – Stakeholder Participation

Attachment C - Timeline

In the Gantt chart, need to add/modify:

Line 3.5 – role of TPAS and ESP

Lines 4, 5 – identify committees by name, change approval to vote

Line 6 – change approval to acceptance

Timing on Gantt chart should to be reviewed, for example:

- Time for input from TOs, other stakeholders, coordination with Control Areas

- Committee actions don’t make sense if study is not completed.