New York Independent System Operator, Inc. 	First Revised Sheet No. 58

FERC Electric Tariff	Superseding Original Sheet No. 58

Original Volume No. 2

to meet Load which was not security constrained.  Out-of-Merit Generation occurs to maintain system reliability or to provide Ancillary Services.

2.136 	Performance Index

An index, described in ISO Procedures, that tracks a Generator’s response to AGC signals from the ISO.

2.137 	Performance Tracking System

A system designed to provide quantitative comparisons of actual values versus expected and forecasted values for Generators and Loads.  This system will be used by the ISO to measure compliance with criteria associated with, but not limited to, the provision of Regulation and Frequency Response Service.

xx.xxx.xx. Available Reserves (ALPHABETIZE)

For purposes of determining the Real-Time Locational Based Marginal Price in any Security Constrained Dispatch interval: the capability of all Suppliers to provide spinning reserves, non-synchronized 10-minute reserves, and 30-minute reserves in that interval,  the quantity of recallable external ICAP energy sales in that interval, and the Load reduction available with a 5% Voltage reduction in that interval.

2.127.2    Persistent Ten Minute Reserves Shortage

For purposes of determining the Real-Time Locational Based Marginal Price, the failure to meet the ten minute Operating Reserves requirement in any Security Constrained Dispatch interval, during an Emergency condition, that may occur after the ISO has (i) started all providers of 30 minute reserve so that they can provide either energy or 10 minute synchronized reserve; (ii) counted as ten-minute reserve those providers that could be started to produce energy or ten-minute synchronized reserves; (iii) recalled its external ICAP energy sales, (iii) activated the Emergency Demand Response Program and requested Load reductions from Special Case Resources and (iv) counted as Operating Reserves the Load reduction available with a 5% Voltage reduction, provided however, that the ISO has determined, pursuant to ISO Procedures, that the failure to meet the ten minute Operating reserves requirement in any Security Constrained Dispatch interval is persistent.

xxx.xxx. Expected Load Reduction (alphabetize)

For purposes of determining the Real-Time Locational Based Marginal Price, the reduction in Load expected be realized in real-time from activation of the Emergency Demand Response Program and from Load reductions requested from Special Case Resources, as established pursuant to ISO Procedures.

2.138 	Point to Point Transmission Service

The reservation and transmission of Capacity and Energy on either a firm or non-firm basis from the Point(s) of Receipt to the Point(s) of Delivery under Part II of the Tariff.

2.139 	Point(s) of Injection (“POI” or “Point of Receipt”)

The point(s) on the NYS Transmission System where Energy, Capacity and Ancillary Services will be made available to the ISO by the delivering party under the ISO OATT or the ISO Services Tariff.  The Point(s) of Injection shall be specified in the Service Agreement.

## Issued by:	William J. Museler, President	Effective:	July 25, 2001

Issued on:	June 6, 2001